Congratulations on your purchase of one of the finest woofers available for your vehicle today.

Woofer technology has not changed much in the past 40 years so the acoustic engineers at Soundstream decided to change all the rules. The Picasso line of sub-woofers where developed specifically for use with the Picasso line of amplifiers using technologies developed from today’s space program.

The Picasso sub-woofers use a black zirconium coating on our cones for not only lower cone distortion but also superior thermal dissipation through the cone generated from the voice coil. We also use a progressive slant surround for tighter and smoother bass extension.

Please take a few moments to fill out the warranty card and review this manual before installing this woofer for proper connections and box sizes.

### Design Features

PCS-10, 12 & 15

Black powder coated stamped basket

UDV cooling technology

Black Zirconium cone

Zinc coated CNC Back plate

Low carbon top & pole piece

High efficiency, Progressive slant surround

Rolled edge rubber gasket

Black progressive roll spider

Gold screw terminals

Dual 4ohm voice coil

Rubber magnet cover

Kapton voice coil

| Model | Description | Freq. Response | Magnet Size | Voice Coil | SPL | X-max | Power RMS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PCS-10 | 10" Multi Purp. | 25Hz - 2kHz | 40 oz. | 2" | 87 dB | 5mm | 200 watts |
| PCS-12 | 12" Multi Purp. | 24Hz - 1.7kHz | 40 oz. | 2" | 89 dB | 5mm | 250 watts |
| PCS-15 | 15" Multi Purp. | 21Hz - 1.5kHz | 50 oz. | 2" | 91 dB | 5mm | 300 watts |

#### Building the Enclosure

- Determine the dimensions of your enclosure.

- Be certain the box dimensions that you have designed will fit in the location you have chosen in your vehicle. Sometimes making a cardboard box with the same outside dimensions is helpful.

- It is recommended to use ¾ or 1 inch thick MDF (medium density fiberboard) for your box.

- Use a “T” square to verify precise right angle cuts before you assemble the box.

- Use high quality wood glue and screws to assemble the box to guarantee an airtight box that will not come apart due to excess vibration and pressure.

- It is recommended for sealed enclosures to stuff the interior of the box about 50 – 75% fiberglass insulation or Dacron fiberfill for increased sound damping and woofer performance.

- For ported enclosures, it is recommended to staple 1” thick fiberglass insulation to the interior walls of your box.

- Use slide on connectors for spade style connectors or bare wire for push and screw terminals. Do not solder the wires to the factory connectors as this may cause damage to the voice coil or tinsel lead and may void your warranty.

### Recommended box sizes

| Model | Sealed Box Size | Ported Box Size | Tune Freq. | # of Ports | Port (dia x length) |
| --- | --- | --- | --- | --- | --- |
| PCS-10 | .75 cubic feet | 1.6 cubic feet | 35 Hz | 1 | 3" x 8.25" |
| PCS-12 | 1.0 cubic feet | 2.15 cubic feet | 32 Hz | 1 | 3" x 6.5" |
| PCS-15 | 3.0 cubic feet | 3.3 cubic feet | 28 Hz | 1 | 3" x 5.8" |

### Recommended wiring

The following page will give you a few different options to match the impedance of the woofers to the impedance capability of the amplifier. Please look and follow the diagrams closely to insure maximum woofer performance.

Amplifier

+

-

2-ohm Load

+

+

Amplifier

+

-

4-ohm

4-ohm

1-ohm Load

4-ohm

4-ohm

4-ohm

4-ohm

-

+

+

+

+

-

-

+

+

-

-

+

+

+

+

-

+

+

Parallel voice coil configuration

Series voice coil configuration

+

-

Amplifier

+

-

4-ohm

4-ohm

4-ohm Load

+

-

+

-

4-ohm

4-ohm

Amplifier

+

-

8-ohm Load

+

-

+

-

4-ohm

4-ohm

Series/Parallel voice coil configuration